1.Draw a simple diagram showing the pin layout of an RJ-45 connector and label the color codes for the T568A and T568B Ethernet wiring standards.

Here’s a simple RJ-45 pin layout diagram with T568A and T568B color codes:

```

RJ-45 (8 pins, clip away, contacts facing you)

Pin:   1   2   3   4   5   6   7   8

---------------------------------

A:    W-G  G  W-O  BL  W-BL  O  W-BR  BR

B:    W-O  O  W-G  BL  W-BL  G  W-BR  BR

Legend:

W-G  = White/Green

G    = Green

W-O  = White/Orange

O    = Orange

BL   = Blue

W-BL = White/Blue

BR   = Brown

W-BR = White/Brown

```

T568A and T568B are identical except green and orange pairs are swapped.

2.Create a table comparing the key specifications of Ethernet standards 10BASE-T, 100BASE-TX, and 1000BASE-T, including speed, cable type, maximum cable length, and connector type.

Here is a clear comparison of the Ethernet standards 10BASE-T, 100BASE-TX, and 1000BASE-T:

| Specification | 10BASE-T | 100BASE-TX | 1000BASE-T (Gigabit Ethernet) |
| --- | --- | --- | --- |
| Speed | 10 Mbps | 100 Mbps | 1000 Mbps (1 Gbps) |
| Cable type | Twisted pair (Cat 3 or higher) | Twisted pair (Cat 5 or higher) | Twisted pair (Cat 5e or Cat 6 recommended) |
| Maximum length | 100 meters | 100 meters | 100 meters |
| Connector type | RJ-45 | RJ-45 | RJ-45 |

If you want, I can also add differences in encoding methods, duplex modes, or real-world performance usage.

3.Given a scenario where you need to connect two laptops directly using Ethernet, explain which cable (straight-through or crossover) you would use and why.<br><br><em><strong>Hint:</strong> Think about how transmit and receive pairs are wired in each cable type.</em>

You would use a crossover Ethernet cable.

This is because a crossover cable swaps the transmit and receive pairs (TX ↔ RX), allowing two similar devices like laptops (both act as hosts) to communicate directly without a switch or router in between.

4.List three real-world devices in your home or college that use Ethernet connections, and for each, specify the likely Ethernet standard (e.g., 100BASE-TX, 1000BASE-T) they support.

- Desktop PC (college lab/home) – likely 1000BASE-T (Gigabit Ethernet)

- Wi-Fi router / modem LAN port – typically 1000BASE-T (Gigabit Ethernet)

- Network printer – often 100BASE-TX (Fast Ethernet) or sometimes 1000BASE-T on newer models