1.Given a network address of 192.168.10.0/24, divide it into 4 equal subnets and write down the subnet mask, network address, and broadcast address for each subnet.

To divide 192.168.10.0/24 into 4 equal subnets, we borrow 2 bits → /26.

Subnet mask (all subnets): 255.255.255.192 (/26)

Each subnet has 64 IPs.

- Subnet 1

- Network: 192.168.10.0/26

- Broadcast: 192.168.10.63

- Subnet 2

- Network: 192.168.10.64/26

- Broadcast: 192.168.10.127

- Subnet 3

- Network: 192.168.10.128/26

- Broadcast: 192.168.10.191

- Subnet 4

- Network: 192.168.10.192/26

- Broadcast: 192.168.10.255

- For one of the subnets you created, assign valid IP addresses to 5 devices (such as a router, two laptops, a printer, and a mobile) and list which IP goes to which device.<br><br><em><strong>Hint:</strong> Make sure not to use the network or broadcast address for any device.</em>

Example subnet: 192.168.10.0/24

Valid host range: 192.168.10.1 – 192.168.10.254 (do not use .0 or .255)

Device IP assignments:

- Router: 192.168.10.1

- Laptop 1: 192.168.10.10

- Laptop 2: 192.168.10.11

- Printer: 192.168.10.20

- Mobile: 192.168.10.30

3.Draw a simple diagram (hand-drawn or digital) showing your subnet plan: label each subnet, the range of IPs, and where each device is connected.

[ Internet ]

|

[ Router ]

192.168.0.1/24

|

-------------------

|                 |

Subnet A (LAN)     Subnet B (Staff WiFi)

192.168.10.0/24      192.168.20.0/24

|                 |

[Switch A]        [Wireless AP]

/   |   \              |

/    |    \         Phones/Laptops

PC1      PC2    Printer

192.168.10.10  .11   .50

|

Subnet C (Servers)

192.168.30.0/24

|

[Switch B]

|

File Server / DB

192.168.30.10

I’ll sketch a simple subnet plan as a clean ASCII diagram. Since you didn’t provide a specific topology, I’ll assume a small office network with 3 subnets (you can tell me yours later and I’ll redraw it exactly).

🧭 Subnet Plan Diagram (Simple Office Network)

[ Internet ]

|

[ Router ]

192.168.0.1/24

|

-------------------

|                 |

Subnet A (LAN)     Subnet B (Staff WiFi)

192.168.10.0/24      192.168.20.0/24

|                 |

[Switch A]        [Wireless AP]

/   |   \              |

/    |    \         Phones/Laptops

PC1      PC2    Printer

192.168.10.10  .11   .50

|

Subnet C (Servers)

192.168.30.0/24

|

[Switch B]

|

File Server / DB

192.168.30.10

📌 Subnet Details

🔹 Subnet A – Office LAN

- Network: 192.168.10.0/24

- Usable IP range: 192.168.10.1 – 192.168.10.254

- Devices:

  - PCs (e.g., PC1, PC2)

  - Printer

  - Connected via Switch A

🔹 Subnet B – WiFi Users

- Network: 192.168.20.0/24

- Usable IP range: 192.168.20.1 – 192.168.20.254

- Devices:

  - Laptops

  - Mobile phones

  - Connected via Wireless Access Point

🔹 Subnet C – Servers

- Network: 192.168.30.0/24

- Usable IP range: 192.168.30.1 – 192.168.30.254

- Devices:

  - File server

  - Database server

  - Internal services

🔁 Routing Note

- Router connects all subnets and enables inter-subnet communication.

- Each subnet is isolated at Layer 3 for security and traffic control.

If you want, I can:

- Convert this into a proper network diagram image

- Match it to your real IP scheme

- Add VLAN IDs + switch port mapping

- Or design a Cisco Packet Tracer version

4.Document step-by-step how you calculated the subnet mask, number of hosts per subnet, and assigned IP addresses for your plan, as if you are explaining it to a friend who is new to subnetting.

I’ll walk you through it the same way I’d explain it to a friend who’s just starting subnetting—step by step, and with the “why” behind each move.

Since you didn’t include a specific network plan, I’ll use a simple example and show you the exact process. You can apply the same logic to your own values later.

🧠 Example scenario

Let’s say we are given:

- Network: 192.168.1.0/24

- Requirement: 4 subnets

- We also need to figure out:

  - subnet mask

  - number of hosts per subnet

  - IP ranges for each subnet

1️⃣ Start with the base network

We begin with:

192.168.1.0/24

- /24 means:

  - 24 bits are network bits

  - 8 bits are available for hosts

So the default subnet mask is:

- 255.255.255.0

2️⃣ Decide how many bits to “borrow”

We need 4 subnets.

Subnetting rule:

Number of subnets = 2ⁿ (where n = borrowed bits)

So we solve:

- 2ⁿ = 4

- n = 2 bits

👉 We borrow 2 bits from the host portion

3️⃣ Calculate the new subnet mask

Original mask: /24

We borrowed 2 bits:

- New prefix = /24 + 2 = /26

Now convert /26 to subnet mask:

Binary last octet:

- 11111100 → that equals 252

So new subnet mask is:

👉 255.255.255.192

4️⃣ Calculate hosts per subnet

Now we see how many host bits are left:

- Original host bits = 8

- Borrowed = 2

- Remaining = 6 host bits

Formula:

Hosts per subnet = 2⁶ − 2

Why minus 2?

- 1 address = network ID

- 1 address = broadcast ID

So:

- 2⁶ = 64

- 64 − 2 = 62 usable hosts per subnet

5️⃣ Find block size (important shortcut)

Block size helps us find IP ranges quickly.

Formula:

Block size = 256 − subnet mask value in last octet

So:

- 256 − 192 = 64

👉 Each subnet jumps by 64 in the last octet

6️⃣ Now assign subnets

We start from 192.168.1.0 and add block size (64 each time)

🔹 Subnet 1

- Network: 192.168.1.0

- Broadcast: 192.168.1.63

- Usable: 192.168.1.1 – 192.168.1.62

🔹 Subnet 2

- Network: 192.168.1.64

- Broadcast: 192.168.1.127

- Usable: 192.168.1.65 – 192.168.1.126

🔹 Subnet 3

- Network: 192.168.1.128

- Broadcast: 192.168.1.191

- Usable: 192.168.1.129 – 192.168.1.190

🔹 Subnet 4

- Network: 192.168.1.192

- Broadcast: 192.168.1.255

- Usable: 192.168.1.193 – 192.168.1.254

7️⃣ How IP assignment works in practice

Once subnets are defined:

- Router interface gets the first usable IP

- Servers often get low-range static IPs

- PCs/laptops usually get DHCP addresses

- Network devices are kept consistent (e.g., .1, .65, .129, .193)

Example:

Subnet 1:

- Router: 192.168.1.1

- Server: 192.168.1.2

- DHCP pool: 192.168.1.10 – 192.168.1.62

🧩 Summary of the logic

- Start with base network (/24)

- Decide required subnets → use 2ⁿ

- Borrow bits → get new prefix (/26)

- Convert to subnet mask

- Calculate hosts → 2^host bits − 2

- Find block size → 256 − mask value

- Step through ranges and assign IPs

If you want, I can also:

- show a VLSM (Variable Length Subnet Masking) version

- or walk through a real exam-style question

- or check your own subnetting plan and fix it step-by-step