1.Log in to Cisco Packet Tracer or any network simulation tool, add a switch, and create two VLANs named 'Sales' and 'Support' with VLAN IDs 10 and 20.

Here’s a short step-by-step answer:

- Open Cisco Packet Tracer and place a Switch (e.g., 2960).

- Click the switch → go to CLI.

- Enter configuration mode:

enable

configure terminal

- Create VLAN 10 (Sales):

vlan 10

name Sales

exit

- Create VLAN 20 (Support):

vlan 20

name Support

exit

- Verify VLANs:

show vlan brief

2.Assign three different switch ports to VLAN 10 (Sales) and two ports to VLAN 20 (Support), then verify the VLAN membership using the show vlan command.<br><br><em><strong>Hint:</strong> Use the interface range command to select multiple ports efficiently.</em>

Here’s how you can do it on a Cisco switch.

1) Enter global configuration mode

enable

configure terminal

2) Create VLANs (if they don’t already exist)

vlan 10

name Sales

exit

vlan 20

name Support

exit

3) Assign 3 ports to VLAN 10 (Sales)

(Example: FastEthernet 0/1–0/3)

interface range fastethernet 0/1 - 3

switchport mode access

switchport access vlan 10

exit

4) Assign 2 ports to VLAN 20 (Support)

(Example: FastEthernet 0/4–0/5)

interface range fastethernet 0/4 - 5

switchport mode access

switchport access vlan 20

exit

5) Verify VLAN membership

show vlan brief

Expected output (example)

You should see something like:

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------

10   Sales                            active    Fa0/1, Fa0/2, Fa0/3

20   Support                          active    Fa0/4, Fa0/5

If you want, I can also show how to verify per-interface using show interface switchport or how to save the configuration permanently.

3.Connect two routers using a serial or Ethernet link in your simulation tool, assign IP addresses to their interfaces, and configure a static route on Router 1 to reach the LAN behind Router 2.

Example (Router-to-Router via Ethernet):

Topology:

- Router1 ↔ Router2 (Ethernet link)

- LAN1 behind Router1

- LAN2 behind Router2

1. IP Addressing

Router1

- G0/0 (to Router2): 192.168.12.1/30

- LAN interface: 192.168.1.1/24

Router2

- G0/0 (to Router1): 192.168.12.2/30

- LAN interface: 192.168.2.1/24

2. Router Configuration

Router1

interface g0/0

ip address 192.168.12.1 255.255.255.252

no shutdown

interface g0/1

ip address 192.168.1.1 255.255.255.0

no shutdown

Router2

interface g0/0

ip address 192.168.12.2 255.255.255.252

no shutdown

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

3. Static Route (on Router1)

To reach LAN behind Router2:

ip route 192.168.2.0 255.255.255.0 192.168.12.2

Result:

Router1 can now reach 192.168.2.0/24 network via Router2.

4.Simulate a scenario where a device in the 'Sales' VLAN tries to ping a device in the 'Support' VLAN and explain why the ping fails, then suggest what configuration would allow inter-VLAN communication.<br><br><em><strong>Hint:</strong> Think about router-on-a-stick or Layer 3 switch concepts.</em>

Scenario:
A PC in the Sales VLAN (e.g., VLAN 10) tries to ping a PC in the Support VLAN (e.g., VLAN 20). The ping fails because both VLANs are separate Layer 2 broadcast domains and cannot communicate directly.

Why it fails:

- VLANs isolate traffic at Layer 2

- No Layer 3 routing exists between VLAN 10 and VLAN 20

- Switch alone (Layer 2) cannot route packets between VLANs

How to fix it (enable inter-VLAN communication):
Configure inter-VLAN routing using either:

- Router-on-a-stick:

  - Create subinterfaces on a router

  - Assign each VLAN an IP gateway

  - Connect switch to router via trunk link

- Layer 3 switch:

  - Enable ip routing

  - Create SVIs (Switch Virtual Interfaces) for each VLAN

  - Assign gateway IPs to each VLAN interface

Result:
Traffic is routed between VLAN 10 and VLAN 20, allowing successful ping.