1.List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each.

| IPv4 | IPv6 |
| --- | --- |
| 32-bit address scheme | 128-bit address scheme |
| Written in dotted decimal (e.g., 192.168.1.1) | Written in hexadecimal with colons (e.g., 2001:db8::1) |
| Supports about 4.3 billion addresses | Supports a vastly larger number of addresses (2¹²⁸) |
| Header is simpler but includes checksum | More efficient header; no header checksum |
| Uses NAT commonly due to address shortage | NAT generally not required |
| Broadcast addressing is supported | No broadcast; uses multicast/anycast instead |

2.Given the following scenario: You have 4 devices (laptop, phone, smart TV, and printer) connected to your home WiFi. Assign a unique static IPv4 address to each device using the 192.168.10.0/24 network, and write down the IP, subnet mask, and default gateway for each device.

Network: 192.168.10.0/24
Subnet mask: 255.255.255.0
Default gateway (all devices): 192.168.10.1

- Laptop: IP 192.168.10.10, Mask 255.255.255.0, Gateway 192.168.10.1

- Phone: IP 192.168.10.11, Mask 255.255.255.0, Gateway 192.168.10.1

- Smart TV: IP 192.168.10.12, Mask 255.255.255.0, Gateway 192.168.10.1

- Printer: IP 192.168.10.13, Mask 255.255.255.0, Gateway 192.168.10.1

3.Calculate the number of valid subnets and hosts per subnet if you use a /27 CIDR subnet mask on a 192.168.1.0 network. List the first three valid subnet address ranges (network address to broadcast address) and the usable host IPs in each.

Given: 192.168.1.0/24 subnetted into /27

Results

- Number of valid subnets: 8

- Hosts per subnet: 32 total → 30 usable hosts

Subnet details (first 3)

1) 192.168.1.0/27

- Network: 192.168.1.0

- Broadcast: 192.168.1.31

- Usable hosts: 192.168.1.1 – 192.168.1.30

2) 192.168.1.32/27

- Network: 192.168.1.32

- Broadcast: 192.168.1.63

- Usable hosts: 192.168.1.33 – 192.168.1.62

3) 192.168.1.64/27

- Network: 192.168.1.64

- Broadcast: 192.168.1.95

- Usable hosts: 192.168.1.65 – 192.168.1.94

If you want, I can list all 8 subnets in a table.

4.You are given the IP address 10.0.5.17 with subnet mask 255.255.255.0. Determine the network address, broadcast address, and the range of valid host addresses for this subnet.<br><br><em><strong>Hint:</strong> Convert the subnet mask to CIDR notation and use bitwise AND for network address calculation.</em>
Subnet mask 255.255.255.0 corresponds to /24 CIDR notation.

Step 1: Find Network Address (bitwise AND)

IP: 10.0.5.17
Mask: 255.255.255.0

Since the last octet is the host portion:

- Network address = 10.0.5.17 AND 255.255.255.0

- Result = 10.0.5.0

Step 2: Broadcast Address

For a /24 network, all host bits are 1:

- Broadcast address = 10.0.5.255

Step 3: Valid Host Range

- First usable host: 10.0.5.1

- Last usable host: 10.0.5.254

Final Answer:

- Network Address: 10.0.5.0

- Broadcast Address: 10.0.5.255

- Valid Host Range: 10.0.5.1 – 10.0.5.254

5.Use an online subnet calculator tool (such as https://www.subnet-calculator.com/) to verify your answer for the previous task. Take a screenshot of your calculation and write one line explaining if your manual answer matched the tool's output.