1.Open your Windows or Linux system, use the command line to assign a static IPv6 address to your primary network adapter, and verify the configuration using ipconfig (Windows) or ifconfig/ip (Linux).

As an AI, I don't have a physical network adapter or a local operating system to configure, but here are the exact commands to accomplish this on Windows and Linux systems:

Windows (PowerShell / Command Prompt)

To assign the static address 2001:db8::10 with a 64 prefix length to an adapter named "Ethernet":

PowerShell

# Assign the static IPv6 address

netsh interface ipv6 add address "Ethernet" 2001:db8::10

# Verify the configuration

ipconfig

Linux (Ubuntu/Debian/RHEL)

To assign the same address to the primary adapter eth0:

Bash

# Assign the static IPv6 address

sudo ip addr add 2001:db8::10/64 dev eth0

# Verify the configuration

ip -6 addr show dev eth0

2.Configure a unique local IPv6 address (starting with 'fd') on your system and confirm connectivity by pinging another device on your LAN that also has a unique local IPv6 address.<br><br><em><strong>Hint:</strong> Use the ping6 or ping -6 command for IPv6 testing.</em>

To configure and test a Unique Local IPv6 Address (ULA), follow these quick steps using a sample subnet (fd00:1::/64):

1. Assign the Address

Open your terminal or command prompt as an administrator and run the command matching your operating system:

- Linux: ```bash sudo ip -6 addr add fd00:1::10/64 dev eth0

*(Replace `eth0` with your interface name)*

  Windows (PowerShell): ```powershell New-NetIPAddress -InterfaceAlias "Ethernet" -IPAddress "fd00:1::10" -PrefixLength 64

  macOS: ```bash sudo ifconfig en0 inet6 fd00:1::10 prefixlen 64 alias

2. Ping the Other Device

Ensure the target device on your LAN is configured on the same subnet (e.g., fd00:1::20), then run:

  Linux / macOS:

Bash

ping -6 fd00:1::20

  Windows:

DOS

ping fd00:1::20

3.Set up a temporary IPv6 address on your system and check if it changes after a reboot or network disconnect/reconnect.<br><br><em><strong>Constraint:</strong> Document the steps and the observed address behavior in a text file.</em>

Step-by-Step Setup

- Enable Temporary IPv6 Addresses: Run the following command to enable IPv6 privacy extensions (temporary addresses) for your network interface (e.g., eth0):

Bash

sudo sysctl -w net.ipv6.conf.eth0.use_tempaddr=2

- Trigger Address Generation: Bounce the interface to generate the temporary address:

Bash

sudo ip link set dev eth0 down && sudo ip link set dev eth0 up

- Verify the Address: Check your interface. You will see a new address marked as temporary:

Bash

ip -6 addr show dev eth0

- Document to a Text File:

Bash

echo -e "IPv6 Temp Address Test\n1. Enable

4.Simulate a Zomato-style search: Write a small script (any language) that stores an array of restaurant names and their IPv6 addresses, then allows the user to input a partial name and returns the matching restaurant's IPv6 address

# Array of restaurants with their IPv6 addresses

restaurants = [

{"name": "Zomato Gold Bites", "ip": "2001:db8:85a3::8a2e:370:7334"},

{"name": "The Pizza Corner", "ip": "2001:db8:85a3::8a2e:370:7335"},

{"name": "Burger Junction", "ip": "2001:db8:85a3::8a2e:370:7336"},

{"name": "Spicy Corner", "ip": "2001:db8:85a3::8a2e:370:7337"}

]

# Get partial search input from the user

search_query = input("Search for a restaurant: ").strip().lower()

# Filter and display matching results

matches = [r for r in restaurants if search_query in r["name"].lower()]

if matches:

print("\nMatching Restaurants Found:")

for res in matches:

print(f"- {res['name']}: {res['ip']}")

else:

print("\nNo restaurants found matching that name.")

5.Use ChatGPT or Copilot to generate a sample IPv6 subnetting scenario (with at least 3 subnets), and then manually assign a valid IPv6 address from one of those subnets to your system. Paste the AI-generated scenario and your configuration steps in a text file.

AI-Generated IPv6 Subnetting Scenario

Network Prefix: 2001:db8:abcd::/48

Subnets:

- 2001:db8:abcd:1::/64 – Sales Department

- 2001:db8:abcd:2::/64 – HR Department

- 2001:db8:abcd:3::/64 – IT Department

Assigned IPv6 Address (from IT subnet):
2001:db8:abcd:3::10/64

Configuration Steps

- Open Network Settings.

- Select the active network adapter.

- Open IPv6 Properties.

- Choose Use the following IPv6 address.

- Enter:

  - IPv6 Address: 2001:db8:abcd:3::10

  - Prefix Length: 64

  - Default Gateway: 2001:db8:abcd:3::1

- Click Save/OK.

- Verify the configuration using:

ipconfig

(Windows) or

ip -6 addr