1.Open Wireshark, capture network traffic while you refresh the Flipkart homepage, and identify one packet each belonging to the Application, Transport, Internet, and Network Access layers by examining their headers

| TCP/IP Layer | Example Protocol | Wireshark Filter |
| --- | --- | --- |
| Application | HTTPS / DNS | http, tls, or dns |
| Transport | TCP | tcp |
| Internet | IPv4/IPv6 | ip or ipv6 |

2.Create a table comparing the OSI and TCP/IP models, listing all layers side-by-side, and for each layer, give a real-world example from any app you use (e.g., WhatsApp message, Zomato order).

| OSI Model Layer | TCP/IP Model Layer | Real-World Example |
| --- | --- | --- |
| Application | Application | Sending a message on WhatsApp |
| Presentation | Application | WhatsApp encrypts and formats the message |
| Session | Application | Maintaining the chat session between users |
| Transport | Transport | TCP ensures the WhatsApp message is delivered correctly |
| Network | Internet | IP routes the message across the internet |
| Data Link | Network Access | Wi-Fi/Ethernet transfers data between devices on the local network |
| Physical | Network Access | Signals travel through cables, fiber optics, or wireless radio waves |

3.Given a screenshot of a TCP segment header (provided by your trainer), label the fields that belong to the Transport layer and explain what each field does in 1-2 lines.<br><br><em><strong>Hint:</strong> Look for fields like Source Port, Destination Port, Sequence Number, etc.</em>

TCP Transport Layer Fields and Functions

- Source Port – Identifies the sending application.

- Destination Port – Identifies the receiving application.

- Sequence Number – Tracks the order of data bytes sent.

- Acknowledgment Number – Confirms receipt of data from the sender.

- Header Length (Data Offset) – Specifies the size of the TCP header.

- Flags (SYN, ACK, FIN, RST, etc.) – Control connection setup, management, and termination.

- Window Size – Controls the amount of data that can be sent before acknowledgment.

- Checksum – Detects errors in the header and data.

- Urgent Pointer – Indicates the location of urgent data.

- Options/Padding – Provide additional TCP features and proper header alignment.

4.Draw a diagram showing how a message sent from your device to Instagram's server passes through each TCP/IP layer, and annotate what happens at each step using simple app-related terms (e.g., 'message split into packets', 'IP address added', etc.).

User types message

│

▼

┌─────────────────────┐

│ Application Layer   │

│ Instagram app       │

│ → Creates message   │

└─────────────────────┘

│

▼

┌─────────────────────┐

│ Transport Layer     │

│ TCP                 │

│ → Message split     │

│   into packets      │

└─────────────────────┘

│

▼

┌─────────────────────┐

│ Internet Layer      │

│ IP                  │

│ → Adds source &     │

│   destination IP    │

│   addresses         │

└─────────────────────┘

│

▼

┌─────────────────────┐

│ Network Access Layer│

│ Wi-Fi/Mobile Data   │

│ → Converts packets  │

│   into signals and  │

│   sends them        │

└─────────────────────┘

│

▼

Instagram Server

(Receives and reassembles

the message)

5.Pick any one packet you captured in Wireshark and write which TCP/IP layer it belongs to, how you identified it, and what real-world action in an app (like sending a WhatsApp message or booking a ticket on BookMyShow) it represents.

Example packet (Wireshark):

I selected an HTTPS packet (TCP segment) from a connection to a web server (e.g., when loading a website).

- TCP/IP Layer: Transport Layer (TCP)

- How I identified it: In Wireshark, the packet details show protocol as TCP, with source/destination ports (e.g., 443 for HTTPS) and flags like SYN/ACK or PSH, ACK, confirming it is a TCP segment carrying encrypted data.

- Real-world action: This packet represents a user action like logging into an app or booking a ticket on BookMyShow, where the app sends secure encrypted data (like login details or seat selection) to the server.