1.Draw a simple network diagram showing three devices (like your laptop, a friend's phone, and a smart TV) connected to a switch, and label which device is in which VLAN (e.g., VLAN 10 for streaming, VLAN 20 for personal devices).

## 1. Simple VLAN Network Diagram

```text

+-----------+

|  Switch   |

+-----------+

/    |    \

/     |     \

/      |      \

Laptop       Phone     Smart TV

VLAN 20      VLAN 20   VLAN 10

(Personal)   (Personal) (Streaming)

```

* VLAN 10 = Streaming devices (Smart TV)

* VLAN 20 = Personal devices (Laptop, Phone)

2.Configure a simulated switch using Cisco Packet Tracer or NetSim: create two VLANs (VLAN 10 and VLAN 20), assign three PCs to VLAN 10 and two PCs to VLAN 20, and verify that PCs in different VLANs cannot ping each other.<br><br><em><strong>Hint:</strong> Use the 'vlan' and 'switchport access vlan' commands in the simulator CLI.</em>

### Switch Configuration

```bash

enable

configure terminal

vlan 10

name STREAMING

vlan 20

name PERSONAL

interface range fa0/1-3

switchport mode access

switchport access vlan 10

interface range fa0/4-5

switchport mode access

switchport access vlan 20

end

```

### PC Assignment

| PC  | Port  | VLAN |

| --- | ----- | ---- |

| PC1 | Fa0/1 | 10   |

| PC2 | Fa0/2 | 10   |

| PC3 | Fa0/3 | 10   |

| PC4 | Fa0/4 | 20   |

| PC5 | Fa0/5 | 20   |

### Verification

```bash

show vlan brief

```

**Ping Test**

* PC1 → PC2 ✅ Success

* PC1 → PC4 ❌ Fail

* PC2 → PC5 ❌ Fail

Different VLANs cannot communicate without routing.

3.In any network simulator, set up a scenario where you move a PC from VLAN 10 to VLAN 20. Show the before and after configuration commands and test connectivity to confirm the change.

Before

```bash

interface fa0/3

switchport mode access

switchport access vlan 10

```

PC3 belongs to VLAN 10.

After

```bash

interface fa0/3

switchport mode access

switchport access vlan 20

```

PC3 now belongs to VLAN 20.

Connectivity Test

Before:

* PC3 → PC1 ✅ Success

* PC3 → PC4 ❌ Fail

After:

* PC3 → PC1 ❌ Fail

* PC3 → PC4 ✅ Success

4.Write a short script or command list that would automate assigning 10 ports on a switch to VLAN 30, using the syntax of your chosen simulator (Packet Tracer, NetSim, or GNS3).<br><br><em><strong>Constraint:</strong> Use a loop or range command if supported by the tool to avoid repeating the same command 10 times.</em>

bash

enable

configure terminal

This uses the `interface range` command, so you do not need to configure all 10 ports individually.